- U S E R   M A N U A L

- 2X2 4K VIDEO WALL CONTROLLER

- Enjoy the vivid world!

- INTRODUCTION

- 2X2 4K VIDEO WALL CONTROLLER:

- The 4K 2x2 video wall controller , support 1-way HDMI or DVI input and 4 HDMI output, the main function is to divide a complete HDMI,DVI HD image signal into 4 blocks and assign to 4 video display unit (such as Rear projection unit, LCD TV, etc.), Complete with 4 HDMI high-definition video display units to form a large dynamic image screen.

- Three splicing modes : 2x2 ;  1x2 ;  1x4。

- FEATURES

- Support 1 DVI or HDMI input

- Input support HDMI1.4 ,HDCP1.4

- Support 4 HDMI outputs, 1 3.5mm audio output

- Input resolution include 1920x1080P 25/30/50/60HZ,3840x2160P 25/30HZ; Output resolution include 1920x1080P 25/30/50/60HZ

- Input format: YUV422 16bit、RGB444 24bit; Output format: RGB444 24bit

- Support RS232 protocol (Net port RJ45)

- Easy to operate, plug and play

- DC12V/2A power adapter

- Physical interface diagram

- 1：  OFF/ON --- OFF/ON                    2：  HDMI IN ---HDMI input

- 3：  HDMI input signal indicator              4：  DVI IN --- DVI input

- 5：  DVI input signal indicator                6：  L/R OUT -- 3.5mm Stereo output

- 7：  SWITCH --- SWITCH                   8：IR --- Infrared receiver

- 9：  RS232 --- RS232 port       10：  HDMI output signal indicator

- 11-14:   OUT PUT1-OUT PUT4 port        15： DC/12V ---DC12V Power Interface

- Product Picture

- Operating and Connecting

- The Package includes:

- 1. 4K 2X2 video wall controller  x1

- 2. Power adapter            x1

- 3. User Manual              x1